Contents

1.	Core Input Layers	2

1.1.	Topology File (network.json)	2

1.2.	Equipment Library (equipment.json)	3

1.3.	Service Request File (requests.json)	3

1.4.	Generalized Master Input Schema	3

2.	Workflow Scenario	3

2.1.	Optical Path Feasibility Workflow: Determine whether a candidate optical path can support a given service (e.g., 100G, 400G).	4

2.2.	Network Design / Dimensioning Workflow: Design a new optical network or upgrade an existing one.	5

2.3.	What-If Analysis Workflow: Evaluate impact of network changes.	6

2.4.	Capacity Upgrade Planning Workflow: Determine how much additional traffic the network can support.	8

2.5.	Transponder Selection Workflow: Select optimal modulation format for a path.	9

2.6.	SLA Validation Workflow: Ensure services meet SLA after deployment.	9

2.7.	Automated CI/CD Network Validation Workflow: Validate network changes automatically before deployment.	10

2.8.	Multi-Vendor Interoperability Workflow: Test performance across mixed vendor equipment.	11

2.9.	Research & Advanced Modeling Workflow: Study nonlinear effects and advanced propagation models.	12

2.10.	Disaster Recovery Simulation Workflow: Evaluate survivability under failure scenarios.	13

Summary Table	13

What GNPy Is Best At	14

GNPy is a physical-layer simulation engine, NOT a full network optimizer, a controller or a traffic engineering tool.

GNPy handles physics, propagation, SNR. User handle optimization, business logic, constraints, automation

All advanced workflows are built by wrapping automation around propagate(), compute_snr()

GNPy is primarily used for optical network physical-layer modeling and planning (OSNR, impairments, reach estimation, feasibility analysis, etc.).

Structured workflows require:

- Goal / Outcome

- Inputs

  - Network topology (JSON format)

  - Fiber parameters (loss, dispersion, nonlinearity)

  - Amplifier models

  - Transponder specs (baud rate, modulation, FEC threshold)

  - Source and destination nodes

- Requirements

  - GNPy installed

  - Valid equipment library

  - Accurate fiber span data

- Steps

  - Load topology.

  - Select candidate route.

  - Compute span-by-span impairments.

  - Calculate OSNR and nonlinear penalties.

  - Compare against transponder threshold.

- Rules / Conditions

  - If OSNR ≥ required OSNR → Path is feasible.

  - If nonlinear penalty exceeds margin → Reject path.

  - If span loss > amplifier capability → Redesign.

- Outputs

  - OSNR report

  - GSNR margin

  - Pass/fail feasibility

  - Span-level metrics

- Tools / Integration

- KPIs / Validation

  - OSNR margin (dB)

  - Q-factor

  - Remaining system margin

# Core Input Layers

## Topology File (network.json)

{
  "nodes": [],
  "links": [],
  "roadms": [],
  "amplifiers": []
}

## Equipment Library (equipment.json)

{
  "Transceivers": {},
  "Fibers": {},
  "Edfa": {},
  "Roadm": {}
}

## Service Request File (requests.json)

{
"source": "A",
"destination": "Z",
"trx_type": "400G",
"spacing": 50e9,
"power": 0
}

## Generalized Master Input Schema

You can define:

{
  "topology": "network.json",
  "equipment": "equipment.json",
  "requests": "requests.json",
  "scenario": "scenario.json",
  "constraints": {
    "min_margin": 3,
    "max_span_loss": 28,
    "max_channels": 96
  }
}

# Workflow Scenario

## Optical Path Feasibility Workflow: Determine whether a candidate optical path can support a given service (e.g., 100G, 400G).

### Inputs

- Network topology (JSON format)

- Fiber parameters (loss, dispersion, nonlinearity)

- Amplifier models

- Transponder specs (baud rate, modulation, FEC threshold)

- Source and destination nodes

### Requirements

- GNPy installed

- Valid equipment library

- Accurate fiber span data

### Steps

- Load topology.

- Select candidate route.

- Compute span-by-span impairments.

- Calculate OSNR and nonlinear penalties.

- Compare against transponder threshold.

### Rules / Conditions

- If OSNR ≥ required OSNR → Path is feasible.

- If nonlinear penalty exceeds margin → Reject path.

- If span loss > amplifier capability → Redesign.

### Outputs

- OSNR report

- GSNR margin

- Pass/fail feasibility

- Span-level metrics

### KPIs

- OSNR margin (dB)

- Q-factor

- Remaining system margin

### Required Functions

load_network()
load_equipment()
compute_path_dijkstra()
create_input_spectral_information()
propagate()
compute_snr()

### Flow

- Load topology + equipment

- Compute path

- Create spectral information

- Propagate signal

- Compute GSNR

- Compare against threshold

## Network Design / Dimensioning Workflow: Design a new optical network or upgrade an existing one.

### Inputs

- Traffic matrix

- Topology constraints

- Available spectrum

- Equipment capabilities

### Requirements

- Equipment library

- Planning constraints (max reach, max channels)

- Design objectives (cost vs performance)

### Steps

- Import topology.

- Generate possible routes.

- Simulate physical impairments.

- Select viable transponders.

- Optimize amplifier placement.

### Rules

- Use shortest path unless OSNR fails.

- Respect spectral spacing constraints.

- Maintain minimum system margin (e.g., 3 dB).

### Outputs

- Bill of materials

- Amplifier configuration

- Channel allocation

- Feasibility map

### KPIs

- Cost per bit

- Spectrum utilization

- Power margin

### Required Functions

load_network()
load_equipment()
compute_path_dijkstra()
compute_constrained_path()
propagate()
compute_snr()

Plus iteration logic (your own wrapper).

You will:

- Loop over traffic matrix

- Evaluate multiple modulation formats

- Adjust amplifier parameters

GNPy provides physics — you provide optimization logic.

## What-If Analysis Workflow: Evaluate impact of network changes.

### Examples:

- Fiber cut

- Adding new channel

- Changing modulation format

### Inputs

- Baseline network model

- Proposed modification

### Requirements

- Snapshot of current configuration

- Scenario comparison logic

### Steps

- Duplicate baseline topology.

- Apply modification.

- Recompute impairments.

- Compare margins.

### Rules

- Maintain service SLA margin.

- Flag services below threshold.

### Outputs

- Before vs after OSNR comparison

- Risk assessment

- Degradation analysis

### Same base functions:

load_network()
modify_network_element()
propagate()
compute_snr()

You programmatically:

- Remove fiber

- Change amplifier gain

- Add channel

Then re-run propagation.

## Capacity Upgrade Planning Workflow: Determine how much additional traffic the network can support.

### Inputs

- Current channel allocation

- Spectrum grid

- Fiber nonlinear parameters

### Requirements

- Accurate nonlinear model enabled

- Channel power settings

### Steps

- Incrementally add channels.

- Recalculate NLI impact.

- Stop when margin drops below acceptable.

### Rules

- Maintain minimum 2–3 dB margin.

- Avoid exceeding amplifier saturation.

### Outputs

- Maximum channel count

- Spectrum efficiency

- Upgrade recommendations

### Key functions:

create_input_spectral_information()
propagate()
compute_snr()

You iteratively:

- Increase number of channels

- Adjust power

- Observe nonlinear penalty

Stop when:

GSNR < threshold

## Transponder Selection Workflow: Select optimal modulation format for a path.

### Inputs

- Candidate path

- Available modulation formats (QPSK, 8QAM, 16QAM, etc.)

- FEC thresholds

### Steps

- Simulate path performance.

- Evaluate each modulation format.

- Rank by spectral efficiency.

### Rules

- Choose highest spectral efficiency meeting OSNR threshold.

- Include aging margin.

### Outputs

- Recommended modulation format

- Expected reach

- Margin report

### Core functions:

propagate()
compute_snr()

Loop over modulation formats:

for modulation in modulation_list:
    threshold = equipment[modulation].snr_required
    if computed_snr > threshold:
        viable

## SLA Validation Workflow: Ensure services meet SLA after deployment.

### Inputs

- Live network topology

- Service paths

- Performance thresholds

### Steps

- Recompute physical layer margins.

- Compare against SLA.

- Flag risks.

### Rules

- SLA requires X dB margin.

- Flag if predicted degradation > tolerance.

### Outputs

- Compliance report

- Risk score

- Recommended mitigations

### Functions:

propagate()
compute_snr()
filter_requests()

Compare:

margin = computed_snr - required_snr

## Automated CI/CD Network Validation Workflow: Validate network changes automatically before deployment.

### Inputs

- Proposed topology change (JSON)

- Baseline model

### Requirements

- GNPy CLI or API

- CI pipeline (e.g., GitHub Actions)

### Steps

- Trigger validation on config change.

- Run GNPy simulation.

- Compare margins.

- Fail build if constraints violated.

### Rules

- No service may drop below threshold.

- Margin reduction must be < X dB.

### Outputs

- Pass/fail validation

- Detailed impairment report

### Use CLI wrapper:

gnpy-path-request
gnpy-transmission-example

Or API:

propagate()
compute_snr()

Exit pipeline if:

margin < minimum_margin

## Multi-Vendor Interoperability Workflow: Test performance across mixed vendor equipment.

### Inputs

- Vendor-specific amplifier specs

- Transponder performance data

- Fiber types

### Steps

- Model heterogeneous equipment.

- Simulate end-to-end performance.

- Identify incompatibilities.

### Rules

- Ensure compatible gain and noise figures.

- Maintain OSNR margins.

### Outputs

- Compatibility matrix

- Risk areas

- Optimization suggestions

## Research & Advanced Modeling Workflow: Study nonlinear effects and advanced propagation models.

### Inputs

- Fiber nonlinear coefficients

- Raman amplification parameters

- Channel grid

### Steps

- Configure advanced physical models.

- Run GN-model simulations.

- Analyze NLI trends.

### Rules

- Validate against lab measurements.

- Adjust power per channel.

### Outputs

- Nonlinear noise curves

- Optimal launch power

- Academic insights

### Use lower-level core modules:

gnpy.core.elements.Fiber
gnpy.core.propagation._nli()
gnpy.core.propagation._ase()

Tune:

- Launch power

- Raman models

- Channel grid

## Disaster Recovery Simulation Workflow: Evaluate survivability under failure scenarios.

### Inputs

- Topology

- Failure scenario list (node, fiber cut)

### Steps

- Remove failed element.

- Recompute routes.

- Validate physical feasibility.

### Rules

- All critical services must reroute successfully.

- Maintain SLA margin.

### Outputs

- Survivability score

- List of impacted services

- Redundancy gaps

### Use:

- remove_edge()
compute_path_dijkstra()
propagate()

- Loop over failure scenarios.

# Summary Table

| Workflow Type | Strategic | Operational | Research | Automation |
| --- | --- | --- | --- | --- |
| Path feasibility | ✔ | ✔ |  |  |
| Network design | ✔ |  |  |  |
| What-if analysis | ✔ | ✔ |  |  |
| Capacity planning | ✔ |  |  |  |
| SLA validation |  | ✔ |  |  |
| CI/CD validation |  | ✔ |  | ✔ |
| Research modeling |  |  | ✔ |  |
| Disaster recovery | ✔ | ✔ |  |  |

# What GNPy Is Best At

GNPy excels in:

- Physical layer simulation

- OSNR prediction

- Nonlinear impairment modeling

- Open optical network planning

- Pre-deployment validation

If you tell me:

- Are you an operator, vendor, researcher, or automation engineer?

- Are you working on brownfield or greenfield networks?

I can design a full production-grade workflow architecture tailored to your use case.